[image: image1.jpg]GENUINE

MG ROVER

PARTS








MGF/TF Heated Glass Window Fitting Instructions.

Your new MG Rover Parts heated glass screen is made to very exact specifications.  Installation usually takes less than one hour from start to finish.  All the fixing positions and rivet/screw holes are pre-punched to facilitate easy and exact fitting to the hood frame.  The removal of the old plastic window and the fitting of the new glass screen are best accomplished where you have plenty of room to move around the back and sides of the car.

Tools Required

Electric drill with 3.5mm drill bit
Philips screwdriver

Hand held Rivet gun

Window Removal

1. From inside the car release the five catches that hold the rear window section to the car.  Ensure the released catches do not foul the rear bar.
2. Undo the zip and remove the end clip so both sides of the zip can be separated.

3. From outside the rear of the car detach the window section sides from the hood by parting the Velcro.  Lift up the rear bar and place a suitable cover underneath to catch the swarf from drilling out the rivets.
4. Drill out the window section rivets and the first two rivets on each side (of the hood section).

5. Remove the metal strip and keep.

6. Remove the plastic window
Heated Glass Window Installation
7. Attach the wires to the window terminals.  The female wire terminal to the window;the longer cable to the drivers’ side

8. Carefully lay the glass on the rear shelf and attach the bottom edge to the rear bar using the rivets provided and the metal strip removed in (5) above.
9. From inside the car lift the glass and attach to the zip section on the car using the central Velcro pad.  Zip both sections together.

10. Lock the five rear bar catches in position.
11. Run the glass window wiring under the parcel shelf carpet and attach to the wiring block located under the corner of the carpet on the passenger side.

12. In the fuse box located to the under right of the steering column fit the heated window relay into the spare relay block (on pre 2000 model year vehicles it is taped to the wiring loom to the right of the fuse panel). Later cars have a relay block in a similar position to the right of the fuses.  2004my vehicles have the new style micro fuses and relays.
13. Insert the heated window switch in the centre console (third blank from left in 2000my onward vehicles and on the right for pre 2000my vehicles) by removing the switch blank.  To do this remove the plastic panel to the front lower side of the centre console (has the interior light) to provide access to the rear of the switch panel (Pre 2000my vehicle - right side,  2000my onward vehicle - left side).

14. Push the blank switch out from behind, disconnect the wiring plug and connect the new switch.  Push whole assembly back into place and re-attach the front lower side panel.
15. Your window is now ready for use.  When using the heated window circuit the engine has to be running.
Page 1 of 1

